# Technical capabilities

## Python work

To design our device, we first needed an exact description of the magnetic field we want to create. We took a proven stellarator design from the open-source Python package pyQSC, chosen because its field keeps the plasma well confined. We then wrote code that turns this design into a formula for the field at any point in space. The design is written in special coordinates that follow the magnetic field lines, so the code also translates it into ordinary coordinates for COMSOL. We checked the result carefully: positions agree with pyQSC's own calculations to within one millimetre, and two independent ways of computing the field strength agree to within 0.2 %. Our simulation software then uses this formula as the target when it works out how to arrange the permanent magnets.

## COMSOL

We have implemented our optimization scheme in a numerical calculator, where we can optimize the orientation a set of permanent magnets, perturbing a field produced by electromagnets into a shape fit for plasma confinement, using gradient based optimization. This allows us to recreate the magnetic fields of stellarators, on smaller simpler models, fit for display and research models. We are then able to extract the geometry of these configurations, on a format fit for creating a CAD model, allowing us to 3D-print housing that holds the magnets in place, which enables the creation of physical platforms fit for research and displays.